# Práctica de laboratorio 2.5.3: Resolución de problemas de la configuración PPP

## Diagrama de topología

## Tabla de direccionamiento

| Dispositivo | Interfaz | Dirección IP | Máscara de subred | Gateway por defecto |
| --- | --- | --- | --- | --- |
| R1 | Fa0/1 | 10.0.0.1 | 255.255.255.128 | N/C |
|  | S0/0/0 | 172.16.0.1 | 255.255.255.252 | N/C |
|  | S0/0/1 | 172.16.0.9 | 255.255.255.252 | N/C |
| R2 | Lo0 | 209.165.200.161 | 255.255.255.224 | N/C |
|  | S0/0/0 | 172.16.0.2 | 255.255.255.252 | N/C |
|  | S0/0/1 | 172.16.0.5 | 255.255.255.252 | N/C |
| R3 | Fa0/1 | 10.0.0.129 | 255.255.255.128 | N/C |
|  | S0/0/0 | 172.16.0.10 | 255.255.255.252 | N/C |
|  | S0/0/1 | 172.16.0.6 | 255.255.255.252 | N/C |
| PC1 | NIC | 10.0.0.10 | 255.255.255.128 | 10.0.0.1 |
| PC3 | NIC | 10.0.0.139 | 255.255.255.128 | 10.0.0.129 |

## Objetivos de aprendizaje

Para completar esta práctica de laboratorio:

- Cablear una red según el diagrama de topología

- Borrar la configuración de inicio y recargar un router al estado por defecto

- Cargar los routers con guiones

- Detectar y corregir errores de red

- Documentar la red corregida

## Escenario

Un ingeniero de redes inexperto configuró los routers de la compañía. Diversos errores en la configuración produjeron problemas de conectividad. El jefe le solicitó al usuario que resuelva y corrija los errores de configuración y que documente su trabajo. Según los conocimientos de PPP y los métodos de prueba estándar, busque y corrija los errores. Asegúrese de que todos los enlaces seriales utilicen la autenticación PPP CHAP y de que todas las redes sean alcanzables.

Tarea 1: Cargar los routers con los guiones suministrados

R1

enable

configure terminal

!

hostname R1

!

!

enable secret class

!

!

!

no ip domain lookup

!

username R2 password 0 cisco

!

!

!

interface FastEthernet0/0

shutdown

duplex auto

speed auto

!

interface FastEthernet0/1

duplex auto

speed auto

!

interface Serial0/0/0

ip address 172.16.0.1 255.255.255.248

no fair-queue

clockrate 64000

!

interface Serial0/0/1

ip address 172.16.0.9 255.255.255.252

encapsulation ppp

!!

router ospf 1

log-adjacency-changes

network 10.0.0.0 0.0.0.127 area 0

network 172.16.0.8 0.0.0.3 area 0

!!

ip classless

!

ip http server

!

!

control-plane

!

banner motd ^CUnauthorized access strictly prohibited and prosecuted to the full extent of the law^C

!

line con 0

exec-timeout 0 0

password cisco

logging synchronous

login

line aux 0

line vty 0 4

password cisco

login

!

end

R2

enable

configure terminal

!

hostname R2

!

!

enable secret class

!

!

no ip domain lookup

!

username R1 password 0 cisco

username R3 password 0 class

!

!

!

interface Loopback0

!

interface FastEthernet0/0

no ip address

shutdown

duplex auto

speed auto

!

interface FastEthernet0/1

shutdown

duplex auto

speed auto

!

interface Serial0/0/0

ip address 172.16.0.2 255.255.255.252

encapsulation ppp

no fair-queue

ppp authentication chap

!

interface Serial0/0/1

ip address 172.16.0.5 255.255.255.252

!

router ospf 1

log-adjacency-changes

network 172.16.0.0 0.0.0.3 area 0

network 172.16.0.4 0.0.0.3 area 0

ip classless

!

ip http server

!

!

control-plane

!

banner motd ^CUnauthorized access strictly prohibited and prosecuted to the full extent of the law^C

!

line con 0

exec-timeout 0 0

password cisco

logging synchronous

login

line aux 0

line vty 0 4

password cisco

login

!

end

R3

enable

configure terminal

!

hostname R3

!

!

enable secret class

!

!

no ip domain lookup

!

username R1 password 0 cisco

!

!

interface FastEthernet0/0

no ip address

shutdown

duplex auto

speed auto

!

interface FastEthernet0/1

duplex auto

speed auto

!

interface Serial0/0/0

ip address 172.16.0.10 255.255.255.252

no fair-queue

clockrate 64000

!

interface Serial0/0/1

encapsulation ppp

!

router ospf 1

log-adjacency-changes

network 10.0.0.128 0.0.0.127 area 0

!

ip classless

!

ip http server

!

!

control-plane

!

banner motd ^CUnauthorized access strictly prohibited and prosecuted to the full extent of the law^C

!

line con 0

exec-timeout 0 0

password cisco

logging synchronous

login

line aux 0

line vty 0 4

password cisco

login

!

end

Tarea 2: Buscar y corregir errores de red

Tarea 3: Documentar la red corregida

Ahora que se corrigieron todos los errores y se probó la conectividad en toda la red, debe documentarse la configuración final de cada dispositivo.

Tarea 4: Limpiar

Borre las configuraciones y recargue los routers. Desconecte y guarde los cables. Para los equipos PC host que normalmente se conectan a otras redes (tal como la LAN de la escuela o Internet), reconecte los cables correspondientes y restablezca las configuraciones TCP/IP.