# Práctica de laboratorio 8.5.2: Resolución de problemas de redes empresariales 2

## Diagrama de topología

## Tabla de direccionamiento

| Dispositivo | Interfaz | Dirección IP | Máscara de subred | Gateway por defecto |
| --- | --- | --- | --- | --- |
| R1 | Fa0/0 | 192.168.10.1 | 255.255.255.0 | N/C |
|  | Fa0/1 | 192.168.11.1 | 255.255.255.0 | N/C |
|  | S0/0/0 | 10.1.1.1 | 255.255.255.252 | N/C |
|  | S0/0/1 | 10.3.3.1 | 255.255.255.252 | N/C |
| R2 | Fa0/1 | 192.168.20.1 | 255.255.255.0 | N/C |
|  | S0/0/0 | 10.1.1.2 | 255.255.255.252 | N/C |
|  | S0/0/1 | 10.2.2.1 | 255.255.255.252 | N/C |
|  | Lo0 | 209.165.200.225 | 255.255.255.224 | 209.165.200.226 |
| R3 | Fa0/1 | N/C | N/C | N/C |
|  | Fa0/1.11 | 192.168.11.3 | 255.255.255.0 | N/C |
|  | Fa0/1.30 | 192.168.30.1 | 255.255.255.0 | N/C |
|  | S0/0/0 | 10.3.3.2 | 255.255.255.252 | N/C |
|  | S0/0/1 | 10.2.2.2 | 255.255.255.252 | N/C |
| S1 | VLAN10 | DHCP |  | N/C |
| S2 | VLAN11 | 192.168.11.2 | 255.255.255.0 | N/C |
| S3 | VLAN30 | 192.168.30.2 | 255.255.255.0 | N/C |
| PC1 | NIC | DHCP |  |  |
| PC2 | NIC | 192.168.11.10 | 255.255.255.0 | 192.168.11.1 |
| PC3 | NIC | 192.168.30.10 | 255.255.255.0 | 192.168.30.1 |
| Servidor TFTP | NIC | 192.168.20.254 | 255.255.255.0 | 192.168.20.1 |

## Objetivos de aprendizaje

Al completar esta práctica de laboratorio, el usuario podrá:

- Cablear una red según el diagrama de topología

- Borrar la configuración de inicio y recargar un router al estado por defecto

- Cargar los routers y switches con los guiones suministrados

- Detectar y corregir todos los errores de red

- Documentar la red corregida

## Escenario

Para esta práctica de laboratorio, no se debe utilizar la protección por contraseña o por inicio de sesión en ninguna línea de consola para evitar que se produzca un bloqueo accidental. Utilice ciscoccna para todas las contraseñas en esta práctica de laboratorio.

Nota: Debido a que esta práctica de laboratorio es acumulativa, se utilizarán todos los conocimientos y las técnicas de resolución de problemas que se hayan adquirido en materiales anteriores para completarla con éxito.

### Requisitos

- S2 es la raíz de spanning tree para la VLAN 11 y S3 es la raíz de spanning tree para la VLAN 30.

- S3 es un servidor VTP con S2 como cliente.

- El enlace serial entre R1 y R2 es Frame Relay.

- El enlace serial entre R2 y R3 utiliza encapsulación HDLC.

- El enlace serial entre R1 y R3 se autentica mediante CHAP.

- R2 debe tener procedimientos seguros de inicio de sesión, ya que es el router extremo de Internet.

- Todas las líneas vty, excepto las que pertenecen a R2, permiten conexiones sólo desde las subredes que se muestran en el diagrama de topología, sin incluir la dirección pública.

- Se debería impedir la suplantación de identidad (spoofing) de la dirección IP de origen en todos los enlaces que no se conectan al resto de los routers.

- Los protocolos de enrutamiento deben utilizarse de manera segura. En esta situación se usa EIGRP.

- R3 no debe poder establecer una conexión telnet a R2 a través del enlace serial conectado en forma directa.

- R3 tiene acceso a las VLAN 11 y 30 a través de su puerto 0/1 Fast Ethernet.

- El servidor TFTP no debería recibir ningún tipo de tráfico que tenga una dirección de origen fuera de la subred. Todos los dispositivos tienen acceso al servidor TFTP.

- Todos los dispositivos de la subred 192.168.10.0 deben poder obtener sus direcciones IP de DHCP en R1. Esto incluye a S1.

- Se debe poder acceder a todas las direcciones que se muestran en el diagrama desde cada dispositivo.

Tarea 1: Cargar los routers con los guiones suministrados

!------------------------------------------

!                  R1

!------------------------------------------

no service password-encryption

!

hostname R1

!

boot-start-marker

boot-end-marker

!

security passwords min-length 6

enable secret ciscoccna

!

ip cef

!

ip dhcp pool Access1

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

!

no ip domain lookup

frame-relay switching

!

username R2 password ciscoccna

username ccna password ciscoccna

!

interface FastEthernet0/0

ip address 192.168.10.1 255.255.255.0

ip access-group Anti-spoofing out

duplex auto

speed auto

no shutdown

!

interface FastEthernet0/1

ip address 192.168.11.1 255.255.255.0

duplex auto

speed auto

no shutdown

!

interface Serial0/0/0

ip address 10.1.1.1 255.255.255.252

encapsulation frame-relay

no keepalive

clockrate 128000

frame-relay map ip 10.1.1.1 201

frame-relay map ip 10.1.1.2 201 broadcast

no frame-relay inverse-arp

frame-relay intf-type dce

no shutdown

!

interface Serial0/0/1

ip address 10.3.3.1 255.255.255.0

!

!

encapsulation ppp

ppp authentication chap

no shutdown

!

interface Serial0/1/0

no ip address

shutdown

clockrate 2000000

!

interface Serial0/1/1

no ip address

shutdown

!

router eigrp 10

passive-interface default

no passive-interface FastEthernet0/0

no passive-interface FastEthernet0/1

no passive-interface Serial0/0/0

no passive-interface Serial0/0/1

network 10.1.1.0 0.0.0.255

network 10.2.2.0 0.0.0.255

network 192.168.10.0 0.0.0.255

network 192.168.11.0 0.0.0.255

no auto-summary

!

ip route 0.0.0.0 0.0.0.0 10.1.1.2

!

ip http server

!

ip access-list standard Anti-spoofing

permit 192.168.10.0 0.0.0.255

deny any

ip access-list standard VTY

permit 10.0.0.0 0.255.255.255

permit 192.168.10.0 0.0.0.255

permit 192.168.11.0 0.0.0.255

permit 192.168.20.0 0.0.0.255

permit 192.168.30.0 0.0.0.255

!

line con 0

exec-timeout 5 0

logging synchronous

line aux 0

line vty 0 4

access-class VTY in

login local

!

end

!------------------------------------------

!                  R2

!------------------------------------------

no service password-encryption

!

hostname R2

!

security passwords min-length 6

enable secret ciscoccna

!

aaa new-model

!

aaa authentication login local_auth local

aaa session-id common

!

ip cef

!

no ip domain lookup

!

username ccna password 0 ciscoccna

!

interface Loopback0

ip address 209.165.200.225 255.255.255.224

ip access-group private in

!

interface FastEthernet0/1

ip address 192.168.20.1 255.255.255.0

ip access-group TFTP out

ip access-group Anti-spoofing in

ip nat outside

no shutdown

!

!

interface Serial0/0/0

ip address 10.1.1.2 255.255.255.252

ip nat inside

encapsulation frame-relay

no keepalive

frame-relay map ip 10.1.1.1 201 broadcast

frame-relay map ip 10.1.1.2 201

no frame-relay inverse-arp

no shutdown

!

interface Serial0/0/1

ip address 10.2.2.1 255.255.255.252

ip access-group R3-telnet in ip nat inside

clockrate 128000

no shutdown

!

!

router eigrp 100

passive-interface default

no passive-interface FastEthernet0/1

no passive-interface Serial0/0/0

no passive-interface Serial0/0/1

no passive interface lo0

network 10.1.1.0 0.0.0.3

network 10.2.2.0 0.0.0.3

network 192.168.20.0 0.0.0.255

network 209.165.200.0 0.0.0.7

no auto-summary

!

ip route 0.0.0.0 0.0.0.0 209.165.200.226

!

no ip http server

ip nat inside source list NAT interface FastEthernet0/0 overload

!

ip access-list standard Anti-spoofing

permit 192.168.20.0 0.0.0.255

deny any

ip access-list standard NAT

permit 10.0.0.0 0.255.255.255

permit 192.168.0.0 0.0.255.255

ip access-list standard private

deny   127.0.0.1

deny   10.0.0.0 0.255.255.255

deny   172.16.0.0 0.15.255.255

deny   192.168.0.0 0.0.255.255

permit any

!

ip access-list extended R3-telnet

deny   tcp host 10.2.2.2 host 10.2.2.1 eq telnet

deny   tcp host 10.3.3.2 host 10.2.2.1 eq telnet

deny   tcp host 192.168.11.3 host 10.2.2.1 eq telnet

deny   tcp host 192.168.30.1 host 10.2.2.1 eq telnet

!

ip access-list standard TFTP

permit 192.168.20.0 0.0.0.255

!

control-plane

!

line con 0

exec-timeout 5 0

logging synchronous

line aux 0

exec-timeout 15 0

logging synchronous

login authentication local_auth

transport output telnet

line vty 0 4

exec-timeout 15 0

logging synchronous

login authentication local_auth

transport input telnet

!

end

!------------------------------------------

!                  R3

!------------------------------------------

no service password-encryption

!

hostname R3

!

security passwords min-length 6

!

no aaa new-model

!

ip cef

!

no ip domain lookup

!

username R1 password ciscoccna

username ccna password ciscoccna

!

interface FastEthernet0/1

no shutdown

!

interface FastEthernet0/1.11

encapsulation dot1Q 11

ip address 192.168.11.3 255.255.255.0

no snmp trap link-status

!

interface FastEthernet0/1.30

encapsulation dot1Q 30

ip address 192.168.30.1 255.255.255.0

ip access-group Anti-Spoofin in

no shutdown

!

!

interface Serial0/0/0

ip address 10.3.3.2 255.255.255.252

encapsulation ppp

ppp authentication pap

!

interface Serial0/0/1

ip address 10.2.2.2 255.255.255.252

no shutdown

!

router eigrp 10

network 10.3.3.0 0.0.0.3

network 10.2.2.0 0.0.0.3

network 192.168.11.0 0.0.0.255

network 192.168.30.0 0.0.0.255

no auto-summary

!

ip classless

!

ip http server

!

ip access-list standard Anti-spoofing

permit 192.168.30.0 0.0.0.255

deny any

ip access-list standard VTY

permit 10.0.0.0 0.255.255.255

permit 192.168.10.0 0.0.0.255

permit 192.168.11.0 0.0.0.255

permit 192.168.20.0 0.0.0.255

permit 192.168.30.0 0.0.0.255

!

!

line con 0

exec-timeout 5 0

logging synchronous

line aux 0

exec-timeout 15 0

logging synchronous

line vty 0 4

access-class VTY out

exec-timeout 15 0

logging synchronous

login local

!

end

!-----------------------------------------

!                S1

!-----------------------------------------

no service password-encryption

!

hostname S1

!

security passwords min-length 6

enable secret ciscoccna

!

no aaa new-model

vtp domain CCNA_Troubleshooting

vtp mode transparent

vtp password ciscoccna

ip subnet-zero

!

no ip domain-lookup

!

no file verify auto

spanning-tree mode pvst

spanning-tree extend system-id

!

vlan internal allocation policy ascending

!

vlan 10

!

interface FastEthernet0/1

switchport access vlan 10

switchport mode access

!

interface FastEthernet0/2

switchport access vlan 10

switchport mode access

!

interface range FastEthernet0/3-24

!

interface GigabitEthernet0/1

shutdown

!

interface GigabitEthernet0/2

shutdown

!

interface Vlan1

no ip address

no ip route-cache

!

interface Vlan10

ip address dhcp

no ip route-cache

!

ip default-gateway 192.168.10.1

ip http server

!

line con 0

exec-timeout 5 0

logging synchronous

line vty 0 4

password ciscoccna

login

line vty 5 15

no login

!

end

!-----------------------------------------

!                S2

!-----------------------------------------

no service pad

service timestamps debug uptime

service timestamps log uptime

no service password-encryption

!

hostname S2

!

security passwords min-length 6

enable secret ciscoccna

!

no aaa new-model

vtp domain CCNA_Troubleshooting

vtp mode Client

vtp password ciscoccna

ip subnet-zero

!

no ip domain-lookup

!

no file verify auto

!

spanning-tree mode mst

spanning-tree extend system-id

spanning-tree vlan 30 priority 4096

!

vlan internal allocation policy ascending

!

interface FastEthernet0/1

switchport access vlan 11

switchport mode access

!

interface FastEthernet0/2

switchport access vlan 11

switchport mode access

!

interface FastEthernet0/3

switchport trunk allowed vlan 11,30

switchport mode trunk

!

interface FastEthernet0/4

switchport trunk allowed vlan 11,30

switchport mode trunk

!

interface range FastEthernet0/5-24

shutdown

!

interface GigabitEthernet0/1

shutdown

!

interface GigabitEthernet0/2

shutdown

!

interface Vlan1

no ip address

no ip route-cache

!

interface Vlan11

ip address 192.168.11.2 255.255.255.0

no ip route-cache

!

ip http server

!

control-plane

!

line con 0

exec-timeout 5 0

logging synchronous

line vty 0 4

password ciscoccna

login

line vty 5 15

no login

!

end

!-----------------------------------------

!                S3

!-----------------------------------------

no service password-encryption

!

hostname S3

!

security passwords min-length 6

enable secret ciscoccna

!

no aaa new-model

vtp domain CCNA_Troubleshooting

vtp mode Server

vtp password ciscoccna

ip subnet-zero

!

no ip domain-lookup

!

no file verify auto

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

spanning-tree vlan 11 priority 4096

vlan internal allocation policy ascending

!

Vlan 11,30

!

interface FastEthernet0/1

switchport trunk allowed vlan 11,30

switchport mode trunk

!

interface FastEthernet0/2

switchport access vlan 30

switchport mode access

!

interface FastEthernet0/3

switchport trunk allowed vlan 11,30

switchport mode trunk

!

interface FastEthernet0/4

switchport trunk allowed vlan 11,30

switchport mode trunk

!

interface range FastEthernet0/5-24

shutdown

!

interface GigabitEthernet0/1

shutdown

!

interface GigabitEthernet0/2

shutdown

!

interface Vlan1

no ip address

no ip route-cache

!

interface Vlan30

ip address 192.168.30.2 255.255.255.0

no ip route-cache

!

ip default-gateway 192.168.30.1

ip http server

!

line con 0

exec-timeout 5 0

logging synchronous

line vty 0 4

password ciscoccna

login

line vty 5 15

no login

!

end

Tarea 2: Buscar y corregir todos los errores de red

Tarea 3: Verificar que se cumpla totalmente con los requisitos

Tarea 4: Documentar la red corregida

Tarea 5: Limpiar

Borre las configuraciones y recargue los routers. Desconecte y guarde los cables. 
Para los equipos PC host que normalmente se conectan a otras redes (tal como la LAN 
de la escuela o Internet), reconecte los cables correspondientes y restablezca las configuraciones TCP/IP.