# Práctica de laboratorio 3.5.3: Resolución de problemas de Frame Relay

## Diagrama de topología

## Tabla de direccionamiento

| Dispositivo | Interfaz | Dirección IP | Máscara de subred | Gateway por defecto |
| --- | --- | --- | --- | --- |
| R1 | Lo0 | 172.18.11.254 | 255.255.255.0 | N/C |
|  | S0/0/0 | 172.18.221.1 | 255.255.255.252 | N/C |
| R2 | Lo0 | 172.18.111.254 | 255.255.255.0 | N/C |
|  | S0/0/1 | 172.18.221.2 | 255.255.255.252 | N/C |

## Objetivos de aprendizaje

Practicar las aptitudes de resolución de problemas de Frame Relay.

## Escenario

En esta práctica de laboratorio el usuario practicará la resolución de problemas en un entorno de Frame Relay mal configurado. Cargue o solicítele al instructor que cargue las configuraciones que aparecen a continuación en los routers. Localice y repare todos los errores en las configuraciones y establezca la conectividad de extremo a extremo. La configuración final debe coincidir con el diagrama de topología y la tabla de direccionamiento. Todas las contraseñas están establecidas en cisco, excepto la contraseña secreta de enable que está establecida en class.

Tarea 1: Preparar la red

Paso 1: Conectar una red que sea similar a la del diagrama de topología.

Paso 2: Borrar todas las configuraciones de los routers.

Paso 3: Importar las configuraciones.

Router 1

!

hostname R1

!

enable secret class

!

no ip domain lookup

!

!

!

!

interface Loopback0

ip address 172.18.11.254 255.255.255.0

!

interface FastEthernet0/0

no ip address

shutdown

duplex auto

speed auto

!

interface FastEthernet0/1

no ip address

shutdown

duplex auto

speed auto

!

interface Serial0/0/1

no ip address

shutdown

no fair-queue

clockrate 125000

!

interface Serial0/0/0

ip address 172.18.221.1 255.255.255.252

encapsulation frame-relay

frame-relay map ip 172.18.221.2 678 broadcast

no frame-relay inverse-arp

no shutdown

!

router eigrp 1

network 172.18.221.0

network 172.18.11.0

no auto-summary

!

!

!

line con 0

password cisco

logging synchronous

line aux 0

line vty 0 4

password cisco

login

!

end

Router 2

!

hostname R2

!

enable secret class

!

no ip domain lookup

!

interface Loopback0

ip address 172.18.111.254 255.255.255.0

!

interface FastEthernet0/0

no ip address

shutdown

duplex auto

speed auto

!

interface FastEthernet0/1

no ip address

shutdown

duplex auto

speed auto

!

interface Serial0/0/0

no ip address

shutdown

no fair-queue

!

interface Serial0/0/1

ip address 172.18.221.2 255.255.255.252

encapsulation frame-relay

clockrate 125000

frame-relay map ip 172.18.221.1 181

no frame-relay inverse-arp

frame-relay lmi-type ansi

!

router eigrp 1

network 172.18.221.0

network 172.18.111.0

no auto-summary

!

!

!

line con 0

password cisco

logging synchronous

line aux 0

line vty 0 4

login

!

end

Switch FR:

!

hostname FR-Switch

!

!

enable secret class

!

!

!

no ip domain lookup

frame-relay switching

!

!

!

!

interface FastEthernet0/0

no ip address

shutdown

duplex auto

speed auto

!

interface FastEthernet0/1

no ip address

shutdown

duplex auto

speed auto

!

interface Serial0/0/0

no ip address

encapsulation frame-relay

no fair-queue

clockrate 125000

frame-relay intf-type dce

frame-relay route 182 interface Serial0/0/1 181

no shutdown

!

interface Serial0/0/1

no ip address

clockrate 125000

encapsulation frame-relay

frame-relay intf-type dce

no shutdown

!

!

!

!

line con 0

password cisco

logging synchronous

line aux 0

line vty 0 4

password cisco

login

!

end

Tarea 2: Resolver los problemas y reparar la conexión Frame Relay entre R1 y R2

Tarea 3: Documentar las configuraciones del router

En cada router, ejecute el comando show run y capture las configuraciones.

Tarea 4: Limpiar

Borre las configuraciones y recargue los routers. Desconecte y guarde los cables. Para los equipos PC host que normalmente se conectan a otras redes (tal como la LAN de la escuela o Internet), reconecte los cables correspondientes y restablezca las configuraciones TCP/IP.